NETWORK TRAFFIC ANALYSIS

Project Report & Technical Reference

Capturing, Analyzing and Detecting Threats in Network Traffic using Wireshark, tcpdump and Snort

Tools Covered:   Wireshark  |  tcpdump  |  Snort IDS

Version 1.0   |   2025   |   Security Engineering Department

Classification: CONFIDENTIAL — INTERNAL USE ONLY

Table of Contents

1.   Introduction and Project Objectives

1.1   Project Goals

1.2   Skills Developed

1.3   Lab Environment Setup

2.   Networking Fundamentals for Traffic Analysis

2.1   OSI Model & Traffic Layers

2.2   Key Protocols

2.3   Traffic Flow Concepts

3.   Tool 1 — Wireshark

3.1   Installation and Setup

3.2   Capture Filters

3.3   Display Filters

3.4   Protocol Dissection and Analysis

3.5   Saving and Exporting Captures

4.   Tool 2 — tcpdump

4.1   Installation and Basic Usage

4.2   Essential tcpdump Commands

4.3   Advanced Filtering and Output

4.4   Reading PCAP Files

5.   Tool 3 — Snort IDS

5.1   Installation and Configuration

5.2   Snort Modes of Operation

5.3   Writing Snort Rules

5.4   Running Snort and Reading Alerts

6.   Traffic Analysis Methodology

6.1   Baseline Normal Traffic

6.2   Anomaly Detection Approach

6.3   Protocol Anomaly Analysis

7.   Attack & Malware Detection

7.1   Port Scan Detection

7.2   DDoS and Flood Detection

7.3   Malware C2 Traffic Patterns

7.4   DNS Tunneling Detection

7.5   ARP Spoofing and MITM Detection

7.6   Ransomware Network Signatures

8.   Hands-On Lab Exercises

8.1   Lab 1 — Basic Packet Capture with Wireshark

8.2   Lab 2 — tcpdump Command Line Analysis

8.3   Lab 3 — Writing and Testing Snort Rules

8.4   Lab 4 — Analyzing a Malware PCAP

9.   Reporting and Documentation

9.1   Traffic Analysis Report Template

9.2   IOC Extraction Format

Appendices

Appendix A   Wireshark Filter Cheat Sheet

Appendix B   tcpdump Command Cheat Sheet

Appendix C   Snort Rule Writing Reference

Appendix D   Common Attack Signatures Table

# 1. Introduction and Project Objectives

Network Traffic Analysis (NTA) is a core discipline in cybersecurity that involves capturing, inspecting, and interpreting data packets as they travel across a network. By studying what flows over the wire, security engineers can identify unauthorized communications, detect malware, spot policy violations, and reconstruct the timeline of a security incident.

This project provides a hands-on, end-to-end guide to network traffic analysis using three industry-standard tools: Wireshark (GUI-based deep inspection), tcpdump (command-line capture), and Snort (signature-based intrusion detection). Together, these tools cover every layer of practical NTA work.

## 1.1 Project Goals

- Understand how to capture live and stored network traffic from various interfaces.

- Apply filters to isolate specific protocols, hosts, ports, and traffic patterns.

- Identify normal versus abnormal traffic behaviour using baseline comparisons.

- Detect signs of attacks including port scans, DDoS, C2 beaconing, and data exfiltration.

- Write effective Snort rules to generate real-time alerts on suspicious activity.

- Produce structured analysis reports suitable for an SOC or incident response team.

## 1.2 Skills Developed

| Skill Area | What You Will Learn | Tool |
| --- | --- | --- |
| Packet Sniffing | Capture raw network frames on any interface, wired or wireless. | Wireshark, tcpdump |
| Traffic Analysis | Decode and interpret TCP, UDP, HTTP, DNS, TLS, and custom protocols. | Wireshark |
| Command-Line Capture | Automate captures, schedule dumps, and filter at wire speed. | tcpdump |
| Intrusion Detection | Deploy signature rules to alert on known attack patterns in real time. | Snort |
| Malware Detection | Identify C2 beaconing, DNS tunneling, and exfiltration in traffic. | Wireshark, Snort |
| IOC Extraction | Pull indicators (IPs, domains, hashes, strings) from traffic captures. | Wireshark, tcpdump |
| Forensic Analysis | Reconstruct sessions, extract files, and replay attacks from PCAPs. | Wireshark |
| Report Writing | Document findings in a structured, evidence-backed analysis report. | All tools |

## 1.3 Lab Environment Setup

IMPORTANT: Always perform packet capture in a controlled lab environment or with explicit written authorization. Unauthorized sniffing is illegal in most jurisdictions.

Recommended lab setup:

- Host OS: Kali Linux 2024 or Ubuntu 22.04 LTS (both ship with Wireshark and tcpdump).

- Virtualization: VirtualBox or VMware — create an isolated internal network (Host-Only adapter).

- Target machine: Metasploitable 2/3 or a Windows VM for generating attack traffic.

- Traffic generation: nmap (port scans), hping3 (flood), Metasploit (payloads), curl/wget (HTTP).

- PCAP samples: Download malware PCAPs from Malware-Traffic-Analysis.net for offline analysis.

| Component | Version | Install Command |
| --- | --- | --- |
| Wireshark | 4.x | sudo apt install wireshark |
| tcpdump | 4.99+ | sudo apt install tcpdump |
| Snort | 2.9.x / 3.x | sudo apt install snort |
| nmap | 7.x | sudo apt install nmap |
| hping3 | 3.x | sudo apt install hping3 |
| Zeek (optional) | 6.x | sudo apt install zeek |

# 2. Networking Fundamentals for Traffic Analysis

Before diving into tools, analysts must understand what they are looking at. Network traffic is structured in layers, and each layer provides different information useful during an investigation.

## 2.1 OSI Model & Traffic Layers

| OSI Layer | Name | Protocols | What to Look For |
| --- | --- | --- | --- |
| Layer 7 | Application | HTTP, DNS, FTP, SMTP, SSH, TLS | URLs, credentials, commands, payloads, C2 patterns |
| Layer 4 | Transport | TCP, UDP, ICMP | Port numbers, flags (SYN, FIN, RST), session establishment |
| Layer 3 | Network | IP, ICMP, ARP | Source/dest IPs, TTL, fragmentation, routing anomalies |
| Layer 2 | Data Link | Ethernet, Wi-Fi | MAC addresses, ARP poisoning, rogue devices |

## 2.2 Key Protocols

| Protocol | Port(s) | Normal Behaviour | Suspicious Behaviour |
| --- | --- | --- | --- |
| HTTP | 80/TCP | Web page requests/responses, REST APIs | Cleartext passwords, malware downloads, C2 over HTTP |
| HTTPS/TLS | 443/TCP | Encrypted web traffic with valid certificate | Self-signed certs, expired certs, unusual SNI values |
| DNS | 53/UDP/TCP | Short queries, legitimate domains, low volume | Long subdomains, high query volume, rare TLDs, tunneling |
| SMTP | 25, 465, 587/TCP | Email delivery, authentication | Spam campaigns, credential harvesting, phishing links |
| SSH | 22/TCP | Interactive logins, SFTP file transfer | Brute force attempts, unusual source IPs, large data transfers |
| FTP | 20, 21/TCP | File transfer, directory listing | Anonymous auth, large file exfiltration, cleartext credentials |
| ICMP | N/A | Echo request/reply (ping), path MTU | ICMP tunneling, flood, covert channel exfiltration |
| ARP | N/A (L2) | IP-to-MAC resolution, broadcast | Gratuitous ARP, ARP storms, ARP spoofing (MITM) |
| SMB | 445/TCP | Windows file sharing, print services | EternalBlue exploits, lateral movement, credential relay |

## 2.3 Traffic Flow Concepts

- Packet: The fundamental unit of network data. Contains headers (metadata) and payload (content).

- Flow / Session: A group of packets sharing the same 5-tuple: source IP, destination IP, source port, destination port, protocol.

- PCAP: Packet Capture file format (.pcap / .pcapng). The universal format used by all major analysis tools.

- Promiscuous Mode: NIC mode that captures ALL frames on the segment, not just those addressed to the capturing host.

- Baseline: A recorded snapshot of normal traffic patterns (volume, protocols, top talkers) used to detect deviations.

# 3. Tool 1 — Wireshark

Wireshark is the world's most widely used network protocol analyser. It provides a graphical interface for capturing live traffic or opening PCAP files, and supports deep dissection of over 3,000 protocols. It is the primary tool for hands-on forensic packet analysis.

## 3.1 Installation and Setup

- Install: sudo apt install wireshark  (select Yes when asked to allow non-root capture)

- Add your user to the wireshark group: sudo usermod -aG wireshark $USER

- Log out and back in, then launch: wireshark &

- Select the correct network interface (e.g., eth0, wlan0, any) from the welcome screen.

- Click the blue shark-fin button or press Ctrl+E to start capturing.

## 3.2 Capture Filters

Capture filters are applied before capture begins and use BPF (Berkeley Packet Filter) syntax. They reduce capture file size by only recording matching traffic.

| Filter | BPF Syntax | Purpose |
| --- | --- | --- |
| Specific host | host 192.168.1.100 | Capture all traffic to/from one IP |
| Specific network | net 192.168.1.0/24 | Capture entire subnet traffic |
| Specific port | port 443 | Capture HTTPS traffic only |
| Port range | portrange 1-1024 | Capture well-known port traffic |
| Protocol | icmp | Capture all ICMP (ping) packets |
| Source host | src host 10.0.0.5 | Traffic originating from one host |
| Destination port | dst port 53 | DNS queries only (outbound) |
| Exclude traffic | not port 22 | Exclude SSH to reduce noise |
| TCP SYN only | tcp[tcpflags] & tcp-syn != 0 | Connection initiation packets |
| Larger than 1000b | greater 1000 | Detect large packet anomalies |

## 3.3 Display Filters

Display filters are applied after capture and use Wireshark's own syntax. They hide/show packets without discarding them. Essential for live analysis.

| Use Case | Display Filter | Description |
| --- | --- | --- |
| HTTP requests only | http.request | Show all HTTP GET/POST requests |
| DNS queries | dns.qry.name | Show all DNS lookups |
| Specific IP conversation | ip.addr == 10.0.0.5 | Filter by IP address |
| Between two hosts | ip.src==10.0.0.1 && ip.dst==10.0.0.2 | Specific conversation pair |
| TCP SYN-ACK | tcp.flags.syn==1 && tcp.flags.ack==1 | Connection handshake responses |
| TCP RST packets | tcp.flags.reset==1 | Connection resets — port scan signature |
| TLS handshakes | tls.handshake | TLS certificate exchanges |
| HTTP 200 OK | http.response.code == 200 | Successful HTTP responses |
| HTTP errors | http.response.code >= 400 | HTTP error responses |
| Large DNS queries | dns && frame.len > 200 | Possible DNS tunneling indicator |
| ICMP unreachable | icmp.type == 3 | Destination unreachable messages |
| ARP spoofing | arp.duplicate-address-detected | Duplicate IP/MAC detected |
| Credentials in HTTP | http contains "password" | Cleartext credential leakage |
| Follow TCP stream | (right-click) Follow > TCP Stream | Reconstruct full session content |

## 3.4 Protocol Dissection and Analysis

### Analysing HTTP Traffic

Use Statistics > HTTP > Requests to see all URLs. Right-click any packet > Follow > HTTP Stream to read the full request/response. Look for:

- POST requests to unusual or external domains (potential data exfiltration).

- User-Agent strings that do not match known browsers (malware beacons).

- Base64-encoded data in request/response bodies.

- HTTP responses with unusual content types (executable downloads).

### Analysing DNS Traffic

Filter with dns. Use Statistics > DNS to see query type distribution. Look for:

- Very long subdomain strings (DNS tunneling: data encoded in subdomains).

- High volume of NXDOMAIN responses (DGA — Domain Generation Algorithm malware).

- Queries to newly registered or suspicious TLDs (.xyz, .top, .tk).

- DNS TXT record queries (common exfiltration channel).

### Analysing TLS/HTTPS Traffic

Even encrypted traffic reveals metadata. Analyse without decryption:

- Certificate Subject and Issuer — look for self-signed or Let's Encrypt certs on suspicious domains.

- Server Name Indication (SNI) — reveals the domain even in HTTPS (filter: tls.handshake.extensions_server_name).

- JA3/JA3S fingerprints — TLS client/server fingerprint that can identify malware families.

- Certificate validity period — malware C2 certs often have very short validity.

## 3.5 Saving and Exporting Captures

# Save capture: File > Save As > capture.pcapng
# Export specific packets: File > Export Specified Packets
# Export objects (files transferred): File > Export Objects > HTTP
# Statistics > Conversations    — top talkers, bytes exchanged
# Statistics > Protocol Hierarchy — breakdown of protocols by %
# Statistics > IO Graph         — visualize traffic volume over time

# 4. Tool 2 — tcpdump

tcpdump is a command-line packet analyser and the standard tool for scripted or server-side captures where a GUI is unavailable. It writes PCAP files that can be opened in Wireshark for deeper analysis. It is indispensable for automated, scheduled, or headless capture workflows.

## 4.1 Installation and Basic Usage

sudo apt install tcpdump        # Install
sudo tcpdump -D                 # List all available interfaces
sudo tcpdump -i eth0            # Capture on eth0 (Ctrl+C to stop)
sudo tcpdump -i any             # Capture on ALL interfaces
sudo tcpdump -i eth0 -n         # Don't resolve hostnames (faster)
sudo tcpdump -i eth0 -nn        # Don't resolve hostnames OR port names
sudo tcpdump -i eth0 -v         # Verbose output
sudo tcpdump -i eth0 -vvv       # Extra verbose (full packet decode)
sudo tcpdump -i eth0 -X         # Print payload in hex AND ASCII
sudo tcpdump -i eth0 -xx        # Print payload in hex (absolute offset)
sudo tcpdump -c 100 -i eth0     # Stop after capturing 100 packets
sudo tcpdump -s 0 -i eth0       # Capture full packet (default truncates)

## 4.2 Essential tcpdump Commands

| Task | Command | Notes |
| --- | --- | --- |
| Capture to file | tcpdump -i eth0 -w capture.pcap | Save to PCAP for later analysis in Wireshark |
| Capture with size limit | tcpdump -i eth0 -w out.pcap -C 100 | Rotate files every 100 MB |
| Capture with time rotation | tcpdump -i eth0 -w cap-%H%M.pcap -G 3600 | New file every 3600 seconds |
| Filter by host | tcpdump -i eth0 host 192.168.1.50 | Only traffic to/from that IP |
| Filter by port | tcpdump -i eth0 port 80 | HTTP traffic only |
| Filter by source IP | tcpdump -i eth0 src 10.0.0.1 | Traffic from specific source |
| Filter by destination port | tcpdump -i eth0 dst port 443 | Outbound HTTPS connections |
| TCP SYN packets only | tcpdump 'tcp[tcpflags] == tcp-syn' | Detect SYN scan activity |
| Exclude SSH (reduce noise) | tcpdump -i eth0 not port 22 | Remove management traffic |
| Combine filters (AND) | tcpdump host 10.0.0.1 and port 53 | DNS from specific host |
| Combine filters (OR) | tcpdump port 80 or port 443 | All web traffic |
| Capture ICMP only | tcpdump -i eth0 icmp | Ping and ICMP tunnel detection |

## 4.3 Advanced Filtering and Output

# Detect SYN flood: high-rate SYN packets from many sources
sudo tcpdump -i eth0 -nn 'tcp[tcpflags] & tcp-syn != 0 and tcp[tcpflags] & tcp-ack == 0'

      
# Detect NULL scan (no flags set — used for stealth OS fingerprinting)
sudo tcpdump -i eth0 'tcp[tcpflags] == 0'

      
# Detect Xmas scan (FIN+URG+PSH set — classic nmap Xmas scan)
sudo tcpdump -i eth0 'tcp[tcpflags] == tcp-fin|tcp-urg|tcp-psh'

      
# Extract HTTP User-Agents from live traffic
sudo tcpdump -i eth0 -A -s 0 'tcp port 80' | grep -i 'user-agent'

      
# Extract DNS query names
sudo tcpdump -i eth0 -l port 53 | grep -oP '(?<=\s)[a-zA-Z0-9.-]+\.[a-z]{2,}'

      
# Capture only large packets (> 1400 bytes) — flag large transfers
sudo tcpdump -i eth0 'greater 1400'

      
# Detect ICMP tunneling (payload > normal ping size)
sudo tcpdump -i eth0 'icmp and greater 200'

## 4.4 Reading PCAP Files

# Read a saved PCAP file
sudo tcpdump -r capture.pcap

      
# Read with verbose decode and no hostname resolution
sudo tcpdump -r capture.pcap -nn -vvv

      
# Apply filters while reading a PCAP
sudo tcpdump -r capture.pcap 'port 53 and udp'

      
# Count packets by protocol (pipe to awk)
sudo tcpdump -r capture.pcap -nn | awk '{print $5}' | sort | uniq -c | sort -rn | head 20

      
# Extract all unique IPs from a PCAP
sudo tcpdump -r capture.pcap -nn | grep -oE '[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+' | sort -u

# 5. Tool 3 — Snort IDS

Snort is an open-source Network Intrusion Detection System (NIDS) developed by Cisco. It inspects network traffic in real time against a library of rules and generates alerts when matches are found. Snort is the industry reference for signature-based IDS and is used in commercial products including Cisco Firepower.

## 5.1 Installation and Configuration

# Install Snort on Ubuntu/Debian
sudo apt update && sudo apt install snort -y

      
# During installation, enter your local network CIDR when prompted
# e.g., 192.168.1.0/24

      
# Verify installation
snort --version

      
# Key configuration file
sudo nano /etc/snort/snort.conf

      
# Set your home network variable (line ~45 in snort.conf)
var HOME_NET 192.168.1.0/24
var EXTERNAL_NET !$HOME_NET

      
# Validate configuration file
sudo snort -T -i eth0 -c /etc/snort/snort.conf

## 5.2 Snort Modes of Operation

| Mode | Command | Description |
| --- | --- | --- |
| Sniffer | snort -v -i eth0 | Display packets on screen. Equivalent to tcpdump. No rules applied. |
| Packet Logger | snort -dev -l /var/log/snort -i eth0 | Log all packets to disk. Useful for offline review. |
| NIDS (Detection) | snort -A console -c /etc/snort/snort.conf -i eth0 | Apply rules and alert in real time. Primary production mode. |
| Offline / PCAP | snort -r capture.pcap -c /etc/snort/snort.conf -A console | Run rules against a saved PCAP file. Excellent for training. |
| Daemon Mode | snort -D -c /etc/snort/snort.conf -i eth0 | Run as background service. Standard for production deployments. |

## 5.3 Writing Snort Rules

Snort Rule Format:   action  protocol  src_ip  src_port  direction  dst_ip  dst_port  (options)

### Rule Anatomy

| Field | Values | Meaning |
| --- | --- | --- |
| Action | alert, log, pass, drop, reject | What Snort does when the rule matches |
| Protocol | tcp, udp, icmp, ip | Transport protocol to match |
| src_ip / dst_ip | IP, CIDR, $HOME_NET, any | Source or destination IP or network |
| src_port / dst_port | 80, 1:1024, !22, any | Port number, range, negation, or any |
| Direction | -> (one way), <> (bidirectional) | Traffic direction arrow |
| msg | "Alert description" | Human-readable alert message (always include) |
| content | "string" or ¦hex bytes¦ | Payload string or hex bytes to match |
| sid | 1000001 and above | Unique Snort rule ID (local rules: 1,000,001+) |
| rev | 1, 2, 3... | Rule revision number (increment on each edit) |

### Example Rules

# Alert on ICMP ping (echo request) from any external host
alert icmp $EXTERNAL_NET any -> $HOME_NET any (msg:"ICMP Echo Request Detected"; itype:8; sid:1000001; rev:1;)

      
# Alert on SSH brute force (connection to port 22 from external)
alert tcp $EXTERNAL_NET any -> $HOME_NET 22 (msg:"Possible SSH Brute Force"; flags:S; threshold:type threshold,track by_src,count 5,seconds 60; sid:1000002; rev:1;)

      
# Detect Nmap SYN scan (many SYN packets, low TTL)
alert tcp any any -> $HOME_NET any (msg:"Possible Nmap SYN Scan"; flags:S; threshold:type threshold,track by_src,count 20,seconds 3; sid:1000003; rev:1;)

      
# Alert on HTTP traffic containing password in URL (credential leakage)
alert tcp any any -> $HOME_NET 80 (msg:"Cleartext Password in HTTP URI"; content:"password="; nocase; http_uri; sid:1000004; rev:1;)

      
# Detect DNS query for suspicious TLD (.xyz)
alert udp $HOME_NET any -> any 53 (msg:"DNS Query to Suspicious TLD .xyz"; content:".xyz"; nocase; sid:1000005; rev:1;)

      
# Detect large ICMP packet (ICMP tunneling indicator)
alert icmp any any -> $HOME_NET any (msg:"Large ICMP Packet - Possible Tunnel"; dsize:>200; sid:1000006; rev:1;)

      
# Alert on SMB EternalBlue exploit attempt (MS17-010)
alert tcp any any -> $HOME_NET 445 (msg:"EternalBlue SMB Exploit Attempt"; content:"|00 00 00 85 ff 53 4d 42|"; depth:8; sid:1000007; rev:1;)

      
# Detect outbound data exfiltration via HTTP POST (large body)
alert tcp $HOME_NET any -> $EXTERNAL_NET 80 (msg:"Large HTTP POST - Possible Exfiltration"; content:"POST"; http_method; dsize:>5000; sid:1000008; rev:1;)

## 5.4 Running Snort and Reading Alerts

# Run in console alert mode (alerts print to terminal)
sudo snort -A console -q -c /etc/snort/snort.conf -i eth0

      
# Run against a PCAP file (great for practice and testing rules)
sudo snort -r suspicious.pcap -c /etc/snort/snort.conf -A console -q

      
# Alert log location (fast alert format)
cat /var/log/snort/alert

      
# Sample alert output format:
# [**] [1:1000003:1] Possible Nmap SYN Scan [**]
# [Priority: 0]
# 03/15-14:22:15.123456 192.168.1.50:54321 -> 192.168.1.1:80
# TCP TTL:64 TOS:0x0 ID:12345 IpLen:20 DgmLen:60
# ******S* Seq: 0xDEADBEEF  Ack: 0x0  Win: 0x1000

      
# Test rules without live traffic (validate rule syntax)
sudo snort -T -c /etc/snort/snort.conf

# 6. Traffic Analysis Methodology

Effective traffic analysis is not just about knowing tool commands — it requires a structured methodology to move from raw packet captures to actionable intelligence. Use the following workflow for any analysis task.

## 6.1 Baseline Normal Traffic

You cannot detect anomalies without knowing what 'normal' looks like. Before any investigation, establish a traffic baseline:

- Capture 1–2 hours of normal working-hours traffic on the monitored segment.

- Open in Wireshark: Statistics > Protocol Hierarchy. Record the % breakdown.

- Statistics > Conversations: identify the top 10 talkers by bytes.

- Statistics > Endpoints: record all active IP and MAC addresses.

- Record average packets/sec and bytes/sec (visible in the status bar).

- Document expected DNS servers, NTP servers, and key infrastructure IPs.

- Save this as your BASELINE.pcap — store securely for future comparison.

## 6.2 Anomaly Detection Approach

| Anomaly Type | What to Look For | Analysis Step |
| --- | --- | --- |
| Volume anomaly | Sudden spike or drop in traffic rate vs. baseline | Statistics > IO Graph — look for rate spikes |
| New endpoint | IP or MAC not seen in baseline | Statistics > Endpoints — compare vs. baseline list |
| New protocol | Protocol % higher than baseline (e.g., ICMP spike) | Statistics > Protocol Hierarchy — delta comparison |
| Unusual port | Traffic on uncommon ports (e.g., 4444, 8080 outbound) | Statistics > Conversations — sort by port |
| Off-hours activity | Traffic from internal IPs at 2–5am when no staff present | View > Time display — check timestamps |
| Geographic anomaly | Connections to unexpected countries (GeoIP lookup) | Right-click IP > GeoIP in Wireshark |
| Beaconing pattern | Periodic connections at fixed intervals to same external IP | Statistics > IO Graph + filter by IP |
| Large DNS response | DNS response larger than 512 bytes (possible tunneling) | Display filter: dns && udp.length > 100 |

## 6.3 Protocol Anomaly Analysis

- TCP Anomalies: RST storms indicate port scanning or refused connections. SYN-only packets with no completion indicate half-open scan. Out-of-order sequence numbers can indicate injection.

- DNS Anomalies: More than 50 NXDOMAIN responses per minute suggests DGA malware. Subdomains longer than 50 characters suggest DNS tunneling. PTR query floods suggest reconnaissance.

- HTTP Anomalies: Periodic HTTP GET requests to the same URL at fixed intervals indicate beaconing. POST requests to unusual IPs indicate exfiltration. Non-browser User-Agents indicate automated tools or malware.

- TLS Anomalies: Certificate CN does not match SNI. Very short certificate validity (< 30 days). Certificate issued by unknown/private CA. Unusual cipher suites or TLS version 1.0/1.1 usage.

# 7. Attack & Malware Detection

## 7.1 Port Scan Detection

Port scans are often the first stage of an attack — the attacker maps open services before exploiting them. Different scan types have distinct packet signatures.

| Scan Type | Packet Signature | Wireshark Filter | Snort Detection |
| --- | --- | --- | --- |
| TCP SYN Scan | Many SYN packets to different ports, no SYN-ACK completion | tcp.flags.syn==1 && tcp.flags.ack==0 | threshold: 20 SYN/3sec from same src |
| TCP Connect Scan | Full 3-way handshake + immediate RST/FIN to many ports | tcp.flags.reset==1 | Track completed + reset connections |
| UDP Scan | UDP packets to closed ports trigger ICMP port unreachable replies | icmp.type==3 && icmp.code==3 | ICMP unreachable flood rule |
| Xmas Scan | TCP packets with FIN+URG+PSH flags set simultaneously | tcp.flags == 0x029 | flags:FPU in Snort rule |
| NULL Scan | TCP packets with no flags set (all zero) | tcp.flags == 0x000 | flags:0 in Snort rule |
| FIN Scan | TCP packets with only FIN flag (no SYN) | tcp.flags == 0x001 | flags:F in Snort rule |

## 7.2 DDoS and Flood Detection

# Wireshark: visualize flood traffic — Statistics > IO Graph
# Look for sudden spike to >10x normal pps rate

      
# tcpdump: capture flood and count packets per second
sudo tcpdump -i eth0 -nn 'icmp or tcp[tcpflags] & tcp-syn != 0' | pv -r > /dev/null

      
# SYN flood signature: many SYN packets, zero ACK responses
# Wireshark display filter for SYN flood analysis:
# tcp.flags.syn==1 and tcp.flags.ack==0

      
# UDP flood: high volume UDP to random ports from spoofed IPs
# Wireshark filter: udp and not dns

      
# ICMP flood (Ping flood / Smurf): extremely high ICMP echo rate
# Wireshark filter: icmp.type==8

      
# HTTP flood (Layer 7 DDoS): normal TCP handshakes + many HTTP GETs
# Wireshark filter: http.request.method=="GET"

## 7.3 Malware C2 Traffic Patterns

Command-and-Control (C2) traffic allows attackers to send instructions to compromised hosts and exfiltrate data. Key patterns to look for:

| C2 Indicator | Description | Detection Method |
| --- | --- | --- |
| Periodic beaconing | HTTP/HTTPS GET request every N seconds to same destination IP | Statistics > IO Graph, filter by suspect IP |
| Low-and-slow traffic | Small packets at regular intervals — evades volume-based detection | Long-duration PCAP + beacon interval analysis |
| Unusual User-Agent | Malware uses fake or generic UA strings like 'Mozilla/4.0' or custom strings | Wireshark: http.user_agent contains "Mozilla/4.0" |
| JA3 fingerprint match | TLS client hello matches known malware TLS fingerprint | Zeek JA3 log + threat intel lookup |
| DGA domains | Rapidly changing destination domains following a DGA pattern | High NXDOMAIN rate + short domain TTL |
| Tor/proxy usage | Traffic to Tor relay IPs or known proxy services | Threat intel feed IP blocklist correlation |
| Encrypted non-443 traffic | TLS/encrypted traffic on non-standard ports (4444, 8443, 1337) | tcp.port != 443 and tls.handshake |
| Large periodic outbound | Regular large HTTP POST / DNS TXT to external IP | Statistics > Conversations, sort by bytes |

## 7.4 DNS Tunneling Detection

DNS tunneling encodes data inside DNS query and response fields to bypass firewalls. It is a stealthy exfiltration technique used by both malware and red teams.

DNS Tunneling Tools: iodine, dnscat2, dns2tcp. Detection requires inspecting query length, volume, entropy, and record types.

# Wireshark: filter all DNS traffic
# Display filter: dns

      
# Look for abnormally long subdomains (tunneling encodes data in subdomains)
# Display filter: dns.qry.name.len > 50

      
# Look for high-entropy query names (random-looking strings = encoded data)
# Visual check: legitimate domains are readable words; tunnel queries look like:
# aGVsbG8gd29ybGQ.example.com  (base64-encoded data)
# 4a4f484e.c2server.xyz        (hex-encoded data)

      
# tcpdump: capture and inspect DNS TXT responses (common exfil vector)
sudo tcpdump -i eth0 -nn 'port 53 and udp' -A | grep -E 'TXT|\.[a-z0-9]{30,}'

      
# Snort rule: detect abnormally long DNS query
alert udp $HOME_NET any -> any 53 (msg:"Long DNS Query - Possible Tunneling"; content:|01 00|; depth:4; dsize:>100; sid:1000009; rev:1;)

## 7.5 ARP Spoofing and MITM Detection

# Wireshark: detect ARP spoofing
# Display filter: arp.duplicate-address-detected
# OR: arp.opcode == 2  (ARP replies — watch for unsolicited replies)

      
# Look for: same IP resolved to two different MAC addresses
# View > Name Resolution > Resolve Physical Addresses

      
# tcpdump: capture ARP traffic only
sudo tcpdump -i eth0 arp -e -n

      
# Red flag output showing ARP poisoning:
# 192.168.1.1 is-at aa:bb:cc:dd:ee:ff   (attacker MAC for gateway)
# 192.168.1.1 is-at 00:11:22:33:44:55   (legitimate gateway MAC)
# Two different MACs for the same IP = ARP spoof confirmed

      
# Snort rule: detect ARP spoofing
alert arp any any -> any any (msg:"Possible ARP Spoofing"; arp.opcode:2; threshold:type both,track by_src,count 50,seconds 10; sid:1000010; rev:1;)

## 7.6 Ransomware Network Signatures

Ransomware generates distinctive network activity before, during, and after encryption:

- Pre-encryption: Connects to C2 for key exchange (often HTTPS, port 443 or 8443).

- Reconnaissance: SMB scanning on port 445 to find shares for lateral spread.

- Key exchange: Reaches out to attacker's public key server (HTTP POST with host ID + public key).

- Shadow copy deletion: No direct network signature but correlates with RPC/DCOM traffic spikes.

- Exfiltration (some variants): Large outbound HTTPS or FTP transfers before encryption begins.

- Post-encryption: HTTP GET to payment site (onion address via Tor, or direct IP).

# 8. Hands-On Lab Exercises

All labs require a virtualized lab environment. Never perform these exercises on production networks or without explicit authorization.

## 8.1 Lab 1 — Basic Packet Capture with Wireshark

### Objective

Capture live network traffic, apply filters, and reconstruct an HTTP session.

### Steps

- Start Wireshark on your lab VM's eth0 interface.

- Open a web browser and visit http://testphp.vulnweb.com (a deliberately vulnerable site).

- Stop the capture after 30 seconds.

- Apply display filter: http — observe all HTTP requests and responses.

- Right-click a GET request > Follow > HTTP Stream — read the full session.

- Apply filter: http.request.method == "POST" — identify any form submissions.

- Use File > Export Objects > HTTP to extract any transferred files.

- Document: all visited URLs, status codes, server version headers, and any credentials visible in plaintext.

### Expected Findings

- HTTP GET requests for HTML, CSS, JS, and image files.

- HTTP response headers revealing server software version (information disclosure).

- Any POST data containing form fields submitted to the server.

## 8.2 Lab 2 — tcpdump Command Line Analysis

### Objective

Use tcpdump to capture, filter, and extract intelligence from command line without a GUI.

### Steps

- From your Kali VM, run an nmap SYN scan against your Metasploitable target: sudo nmap -sS 192.168.1.X

- Simultaneously capture the scan traffic: sudo tcpdump -i eth0 -w lab2_scan.pcap 'host 192.168.1.X'

- Stop both after 60 seconds.

- Read back the PCAP: sudo tcpdump -r lab2_scan.pcap -nn | head 50

- Extract all unique destination ports contacted: sudo tcpdump -r lab2_scan.pcap -nn | awk '{print $5}' | grep -oE '\.[0-9]+:' | sort -u

- Count how many SYN packets were sent: sudo tcpdump -r lab2_scan.pcap 'tcp[tcpflags] == tcp-syn' | wc -l

- Open the PCAP in Wireshark and verify your findings visually.

### Expected Findings

- Thousands of SYN packets to sequential destination ports.

- RST responses from closed ports, SYN-ACK from open ports.

- A clear map of open services on the target.

## 8.3 Lab 3 — Writing and Testing Snort Rules

### Objective

Write custom Snort rules to detect specific attack traffic and validate them against a PCAP.

### Steps

- Download a sample attack PCAP from Malware-Traffic-Analysis.net (any 2024 malware sample).

- Open the PCAP in Wireshark and identify: attacker IP, victim IP, protocol used, suspicious content strings.

- Write a Snort rule targeting the most distinctive characteristic (content string, protocol, port).

- Add rule to /etc/snort/rules/local.rules

- Test against the PCAP: sudo snort -r sample.pcap -c /etc/snort/snort.conf -A console -q

- Verify the alert fires. Tune if needed (adjust content, add nocase, adjust dsize).

- Document the rule, what it detects, and the false positive rate.

## 8.4 Lab 4 — Analyzing a Malware PCAP

### Objective

Perform a full threat analysis on a malware PCAP and produce an IOC report.

# Download: https://www.malware-traffic-analysis.net/2024/index.html
# Pick any recent sample, download the .pcap.zip, password is typically: infected

      
# Step 1: Get an overview
# Wireshark: Statistics > Protocol Hierarchy
# Wireshark: Statistics > Conversations (sort by Bytes)
# Wireshark: Statistics > Endpoints

      
# Step 2: Identify C2 traffic
# Filter: http.request or dns or tcp.flags.syn==1

      
# Step 3: Extract IOCs
# - All external IPs contacted
# - All DNS queries made
# - All HTTP URIs and User-Agents
# - Any files transferred (File > Export Objects > HTTP)
# - Certificate information from TLS handshakes

      
# Step 4: Run through Snort
sudo snort -r malware_sample.pcap -c /etc/snort/snort.conf -A console -q

      
# Step 5: Cross-reference IOCs with threat intel
# VirusTotal: https://www.virustotal.com
# AbuseIPDB:  https://www.abuseipdb.com
# URLhaus:    https://urlhaus.abuse.ch

# 9. Reporting and Documentation

Every traffic analysis engagement must produce a structured written report. Raw PCAPs alone are not a deliverable — findings must be translated into actionable intelligence that non-technical stakeholders can act on.

## 9.1 Traffic Analysis Report Template

| Section | Contents |
| --- | --- |
| Executive Summary | 1 paragraph: what was found, severity, business impact, recommended action. |
| Scope & Methodology | What interfaces were captured, duration, tools used, total packets/bytes analysed. |
| Findings | Each finding has: description, evidence (packet numbers, screenshots), severity (Critical/High/Med/Low). |
| IOC List | Structured table of all indicators: IPs, domains, hashes, URLs, user-agents. |
| Attack Timeline | Chronological reconstruction of events with timestamps from PCAP. |
| Root Cause | How the attacker gained access. What vulnerability or weakness was exploited. |
| Recommendations | Firewall rules to add, patches to apply, configurations to change, monitoring to deploy. |
| Appendix | Raw Snort alert output, PCAP file hashes (SHA256), evidence file list. |

## 9.2 IOC Extraction Format

Use the following structured table to document all extracted indicators:

| IOC Type | Value | First Seen | Last Seen | Confidence | Context |
| --- | --- | --- | --- | --- | --- |
| IP Address | 185.220.101.45 | 2025-03-15 14:22:01 | 2025-03-15 14:55:33 | High | C2 server — periodic beacon every 60s |
| Domain | update-service.xyz | 2025-03-15 14:22:05 | 2025-03-15 14:22:05 | High | DNS query for C2 domain. NXDOMAIN initially. |
| URL | http://185.220.101.45/gate.php | 2025-03-15 14:23:10 | 2025-03-15 14:54:00 | High | HTTP POST — malware check-in beacon |
| User-Agent | Mozilla/4.0 (compatible; MSIE 6.0) | 2025-03-15 14:23:10 | 2025-03-15 14:54:00 | Medium | Legacy UA string used by Emotet malware family |
| File Hash (MD5) | d41d8cd98f00b204e9800998ecf8427e | 2025-03-15 14:30:00 | — | High | PE executable downloaded via HTTP GET /payload.exe |

# Appendix A — Wireshark Filter Cheat Sheet

| Filter | Wireshark Syntax |
| --- | --- |
| CAPTURE FILTERS (BPF — set before capture) |  |
| All traffic from/to IP | host 192.168.1.1 |
| All traffic on subnet | net 192.168.1.0/24 |
| Specific port | port 80 |
| Source host only | src host 10.0.0.1 |
| Exclude port | not port 22 |
| ICMP only | icmp |
| TCP SYN only | tcp[tcpflags] == tcp-syn |
| DISPLAY FILTERS (Wireshark syntax — applied after capture) |  |
| All HTTP requests | http.request |
| Specific IP (either direction) | ip.addr == 10.0.0.1 |
| Between two IPs | ip.src==10.0.0.1 && ip.dst==10.0.0.2 |
| DNS queries | dns.qry.name |
| TLS handshake | tls.handshake |
| HTTP response code | http.response.code == 200 |
| TCP SYN + ACK | tcp.flags.syn==1 && tcp.flags.ack==1 |
| TCP RST | tcp.flags.reset==1 |
| Large DNS (tunneling) | dns && frame.len > 200 |
| Cleartext passwords | http contains "password" |
| ARP spoof detection | arp.duplicate-address-detected |
| Follow TCP stream | Right-click packet > Follow > TCP Stream |

# Appendix B — tcpdump Command Cheat Sheet

| Purpose | Command |
| --- | --- |
| CAPTURE |  |
| Capture on interface | sudo tcpdump -i eth0 -nn |
| Capture all interfaces | sudo tcpdump -i any -nn |
| Save to PCAP file | sudo tcpdump -i eth0 -w output.pcap |
| Full packet content | sudo tcpdump -i eth0 -s 0 -X |
| Stop after N packets | sudo tcpdump -i eth0 -c 1000 |
| FILTERS |  |
| Filter by host | tcpdump host 192.168.1.1 |
| Filter by port | tcpdump port 443 |
| Filter by protocol | tcpdump icmp |
| Exclude traffic | tcpdump not port 22 |
| SYN packets only | tcpdump 'tcp[tcpflags] == tcp-syn' |
| No flags (NULL scan) | tcpdump 'tcp[tcpflags] == 0' |
| READING PCAPs |  |
| Read a PCAP | tcpdump -r capture.pcap -nn |
| Filter while reading | tcpdump -r capture.pcap 'port 53' |
| Extract unique IPs | tcpdump -r file.pcap -nn ¦ grep -oE '[0-9]+\.[0-9]+\.[0-9]+\.[0-9]+' ¦ sort -u |
| Extract User-Agents | tcpdump -A -r file.pcap 'port 80' ¦ grep 'User-Agent' |

# Appendix C — Snort Rule Writing Reference

| Rule Option | Syntax | Description |
| --- | --- | --- |
| DETECTION OPTIONS |  |  |
| content | content:"string"; | Match ASCII string in payload |
| content (hex) | content:"¦DE AD BE EF¦"; | Match hex bytes in payload |
| nocase | nocase; | Case-insensitive content match |
| depth | depth:20; | Only search first N bytes of payload |
| offset | offset:4; | Start searching at byte offset N |
| pcre | pcre:"/regex/i"; | Perl-compatible regex match |
| flags | flags:S; | TCP flag match (S=SYN, A=ACK, F=FIN, R=RST) |
| itype | itype:8; | ICMP type (8=echo request, 0=echo reply) |
| dsize | dsize:>200; | Match on payload size |
| THRESHOLD / RATE OPTIONS |  |  |
| threshold | threshold:type threshold,track by_src,count 5,seconds 60; | Alert after N matches in T seconds from same source |
| detection_filter | detection_filter:track by_src,count 10,seconds 5; | Snort 3 rate filter — suppress until threshold met |
| METADATA OPTIONS |  |  |
| msg | msg:"Alert Name"; | Human-readable alert description (REQUIRED) |
| sid | sid:1000001; | Unique rule ID — local rules start at 1,000,001 |
| rev | rev:1; | Rule revision — increment on every change |
| priority | priority:1; | Alert priority 1 (high) to 3 (low) |
| classtype | classtype:trojan-activity; | Classification for grouping/reporting |

# Appendix D — Common Attack Signatures Table

| Attack | Protocol | Key Signature | Wireshark Filter | Severity |
| --- | --- | --- | --- | --- |
| TCP SYN Scan (nmap) | TCP | Many SYN, no completions | tcp.flags==0x002 && !tcp.flags.ack==1 | High |
| NULL Scan | TCP | TCP flags all zero | tcp.flags==0x000 | Medium |
| Xmas Scan | TCP | FIN+URG+PSH set | tcp.flags==0x029 | Medium |
| SYN Flood (DDoS) | TCP | Massive SYN rate, no ACK | tcp.flags.syn==1 && tcp.flags.ack==0 | Critical |
| ICMP Flood | ICMP | High rate echo requests | icmp.type==8 | High |
| ARP Spoofing | ARP | Duplicate ARP replies | arp.duplicate-address-detected | Critical |
| DNS Tunneling | DNS | Long subdomains, TXT queries | dns.qry.name.len > 50 | High |
| C2 Beaconing | HTTP/S | Periodic requests, fixed interval | http.request.uri contains "/gate" | Critical |
| Data Exfiltration | HTTP/FTP | Large outbound transfer | tcp.len > 5000 && ip.dst != HOME | Critical |
| EternalBlue (MS17-010) | SMB | Specific SMB negotiation bytes | smb2 && tcp.dstport==445 | Critical |
| Mimikatz (LSASS dump) | SMB/RPC | DRSUAPI RPC calls post-compromise | dcerpc.cn_bind_ack | Critical |
| ICMP Tunneling | ICMP | ICMP echo payload > 100 bytes | icmp && data.len > 100 | High |

— END OF DOCUMENT —

This document is reviewed quarterly. All changes must be approved by the Security Engineering Lead.